Detailed Technical Handout (Deep Dive)

Topics Covered

- Azure Virtual Networks (VNet)

- IP Addressing & Address Spaces

- Private IP Address Allocation

- Regions & Subscriptions

- Azure DNS

- VNet Peering

- Gateway Transit & Hub-and-Spoke

- Service Chaining

- Traffic Routing in Azure, Forced Tunneling, Forced Tunneling

- Azure NAT Gateway

1. What is an Azure Virtual Network (VNet)?

An Azure Virtual Network (VNet) is a logically isolated private network within Azure that allows Azure resources (VMs, App Services, AKS, PaaS endpoints) to securely communicate with:

- Each other

- The Internet

- On-premises networks

- Other VNets (same or different regions)

Why VNets Exist

Azure is a multi-tenant cloud. VNets provide:

- Isolation

- IP address control

- Routing boundaries

- Security enforcement

A VNet behaves like a traditional enterprise network, but is:

- Software-defined

- Highly scalable

- Region-scoped

2. Azure Virtual Network – Core Capabilities

Communication Models

| Scenario | Description |
| --- | --- |
| Internet | Outbound/inbound traffic via public IP, NAT, LB |
| Azure-to-Azure | Private communication inside or across VNets |
| Hybrid | VPN / ExpressRoute to on-prem |
| Filtering | NSG, ASG, Azure Firewall |
| Routing | System routes, UDRs, Route Server |

3. Virtual Network Address Space (IP Planning)

RFC 1918 Private IP Ranges

Azure VNets must use private IP ranges:

| Range | CIDR |
| --- | --- |
| 10.0.0.0 – 10.255.255.255 | 10/8 |
| 172.16.0.0 – 172.31.255.255 | 172.16/12 |
| 192.168.0.0 – 192.168.255.255 | 192.168/16 |

Azure-Reserved IP Addresses (Critical Exam Topic)

Azure reserves 5 IPs per subnet:

| IP | Purpose |
| --- | --- |
| x.x.x.0 | Network address |
| x.x.x.1 | Default gateway |
| x.x.x.2-3 | Azure DNS mapping |
| x.x.x.255 | Broadcast |

❗ Design Impact
A /29 subnet gives only 3 usable IPs → often too small.

4. Subnets – Logical Segmentation

A subnet is a routing and security boundary inside a VNet.

Why Subnets Matter

- Attach NSGs

- Associate Route Tables

- Deploy service-specific resources (GatewaySubnet, AzureFirewallSubnet)

Key Rules

- Subnet CIDRs must not overlap

- Cannot resize subnet downward once resources exist

- Some services require dedicated subnets

5. Private IP Address Allocation

Allocation Methods

| Method | Behavior |
| --- | --- |
| Dynamic | Azure assigns next available IP |
| Static | You choose the IP |

Best Practices

- Use static IPs for:

  - Domain controllers

  - DNS servers

  - Firewalls

- Use dynamic IPs for:

  - Application workloads

  - Scale-out services

6. Regions & Subscriptions

- VNets are region-scoped

- VNets cannot span regions

- Connectivity across regions requires:

  - Global VNet Peering

  - VPN / ExpressRoute

Subscription boundaries:

- VNets can peer across subscriptions

- Even across different tenants

7. Public IP Services – Deep Dive

What is a Public IP in Azure?

A Public IP resource represents a routable Internet IP that can be attached to:

- VM NIC

- Load Balancer

- Application Gateway

- VPN Gateway

Public IP SKUs (VERY IMPORTANT)

| Feature | Basic | Standard |
| --- | --- | --- |
| Allocation | Static / Dynamic | Static only |
| Security | Open by default | Secure by default |
| NSG required | Optional | Mandatory |
| Zone support | ❌ | ✅ |
| Recommended | ❌ | ✅ |

❗ Production Rule
👉 Always use Standard SKU

8. Bring Your Own IP (BYOIP)

Used by enterprises that:

- Own public IP ranges

- Want to retain email / IP reputation

- Must comply with allowlists

3-Phase Process

- Validation – Prove ownership

- Provision – Upload prefix

- Commission – Advertise via Azure

9. DNS in Azure – Name Resolution Design

Public DNS vs Private DNS

| Type | Use Case |
| --- | --- |
| Public DNS | Internet-facing services |
| Private DNS | Internal VMs & PaaS |

Azure DNS is authoritative, not recursive.

10. Azure DNS Zones & Delegation

DNS Zone

- Container for records (A, CNAME, MX, TXT…)

- One zone per domain (example: contoso.com)

Delegation Flow

- Register domain at registrar

- Create DNS zone in Azure

- Update registrar NS records → Azure NS

11. Private DNS Zones (Highly Tested Topic)

What They Solve

- Internal DNS without custom DNS servers

- Auto-registration of VM hostnames

- VNet-to-VNet resolution

Key Behavior

- DNS queries resolved only inside linked VNets

- Not reachable from Internet

12. Special IP: 168.63.129.16 (VERY IMPORTANT)

This Azure-owned virtual IP enables:

- DHCP

- Azure DNS

- Load balancer health probes

- VM agent communication

- Platform heartbeat

❗ Never block this IP in NSGs or firewalls.

13. VNet Peering – Cross-VNet Connectivity

What VNet Peering Does

- Connects two VNets privately

- Uses Microsoft backbone

- No gateways required

Key Rules

- Non-transitive

- No overlapping IP ranges

- Supports cross-subscription & cross-tenant

14. Gateway Transit (Hub-and-Spoke)

Allows:

- Spoke VNets to use hub VPN gateway

- Centralized hybrid connectivity

Typical Design

- Hub: VPN / ExpressRoute

- Spokes: Workloads

- UDRs enforce routing

15. Service Chaining

Service chaining forces traffic through:

- Firewall

- NVA

- Inspection appliances

Implemented using:

- User-Defined Routes (UDRs)

- Next hop = Virtual Appliance

16. Virtual Network Traffic Routing

Route Types

| Route Type | Description |
| --- | --- |
| System routes | Auto-created by Azure |
| Default route | 0.0.0.0/0 → Internet |
| UDRs | Custom routing |

Troubleshooting Tool

- Effective Routes (NIC level)

- Network Watcher

17. Forced Tunneling

Forces all Internet traffic via on-prem firewall.

How It Works

- UDR: 0.0.0.0/0 → VPN Gateway

- Requires route-based VPN

- On-prem device must accept default route

18. Azure Route Server

Why It Exists

- Dynamic routing with NVAs

- No manual UDR updates

- Uses BGP

Requirements

- Dedicated RouteServerSubnet

- NVA must support BGP

19. Azure Virtual Network NAT

Problem It Solves

- Outbound Internet access

- Without public IPs on VMs

- Without load balancers

Key Benefits

- Static outbound IP

- Fully managed

- Highly scalable

20. NAT vs Load Balancer (Exam Comparison)

| Feature | NAT Gateway | Load Balancer |
| --- | --- | --- |
| Inbound | ❌ | ✅ |
| Outbound | ✅ | ✅ |
| Static IP | ✅ | ✅ |
| Use case | Egress only | Ingress + Egress |

21. Design Summary (Architect View)

✔ Plan IP ranges early
✔ Use Standard SKU everywhere
✔ Centralize security in hub
✔ Prefer Private Endpoints + Private DNS
✔ Use UDRs + Firewall for inspection
✔ NAT Gateway for outbound-only workloads

22. Recommended Lab Flow (Aligned to PPT)

- Create VNets & Subnets

- Add NSGs

- Configure Public IP (Standard)

- Implement Private DNS

- Peer VNets (Global)

- Add UDRs & Forced Tunneling

- Deploy NAT Gateway

- Validate with Network Watcher