Handout – Design and Implement Azure ExpressRoute

1. What is Azure ExpressRoute?

Azure ExpressRoute provides private, dedicated Layer-3 connectivity between your on-premises network and Microsoft’s global network, bypassing the public internet.

Key Characteristics

- Private connectivity (not encrypted by default, but private)

- Uses BGP for dynamic routing

- High availability with dual Microsoft edge routers

- Enterprise-grade SLA-backed connectivity

- Supports 50 Mbps to 100 Gbps

Why ExpressRoute Exists

Internet VPNs are:

- Best-effort

- Subject to congestion

- Less predictable

ExpressRoute solves this by:

- Providing predictable latency

- Ensuring consistent throughput

- Supporting mission-critical workloads

2. ExpressRoute Capabilities (Deep Dive)

Core Capabilities

| Capability | Explanation |
| --- | --- |
| Layer-3 Connectivity | BGP-based routing between on-prem and Azure |
| Built-in Redundancy | Dual Microsoft routers + dual provider links |
| Global Reach | Connect on-prem sites via Microsoft backbone |
| Multiple Bandwidths | 50 Mbps → 100 Gbps |
| SLA | Up to 99.95% availability |

Billing Models

- Unlimited Data – flat monthly fee

- Metered Data – outbound charged per GB

3. ExpressRoute Use Cases

Common Enterprise Scenarios

- Hybrid cloud data centers

- SAP / ERP workloads

- Large-scale backups

- Disaster recovery replication

- High-frequency financial transactions

Why Enterprises Prefer ExpressRoute

- Stable latency

- Predictable performance

- Regulatory compliance (private routing)

- MPLS integration

4. ExpressRoute Connectivity Models

Connectivity Options

- Private Peering

  - On-prem ↔ Azure VNets

  - Most commonly used

- Microsoft Peering

  - On-prem ↔ Microsoft SaaS (M365, Dynamics)

  - Requires public IP ownership

- Global Reach

  - On-prem ↔ On-prem via Microsoft backbone

5. Design Considerations for ExpressRoute

Key Design Decisions

- Single vs dual circuits

- Local vs Standard vs Premium SKU

- Gateway SKU sizing

- BGP ASN planning

- Coexistence with VPN

High Availability Best Practices

- Dual physical circuits (different providers)

- Separate peering locations

- Zone-redundant gateways (AZ SKUs)

6. Bidirectional Forwarding Detection (BFD)

What is BFD?

A fast failure detection protocol that:

- Detects link failure in milliseconds

- Works alongside BGP

- Improves convergence time

Key Points

- Enabled by default on ExpressRoute

- Must be configured on:

  - Primary on-prem router

  - Secondary on-prem router

- Linked to BGP session

7. Encrypting Traffic over ExpressRoute

Important Concept

ExpressRoute is private, NOT encrypted by default

Encryption Options

- IPsec VPN over ExpressRoute

- Common in regulated industries

Coexistence Design

- ExpressRoute → Primary path

- Site-to-Site VPN → Failover path

8. ExpressRoute SKUs and Tiers

| SKU | Scope |
| --- | --- |
| Local | 1–2 nearby Azure regions |
| Standard | All regions in geopolitical area |
| Premium | Global + >4K routes + >10 VNets |

Premium Add-on

- Required for Global Reach

- Required for global VNet connectivity

9. Choosing Peering Location & Circuit

Selection Criteria

- Physical proximity

- Service provider availability

- Latency requirements

- Compliance needs

Circuit Decisions

- Bandwidth (can increase, not decrease)

- Metered vs Unlimited

- Provider redundancy

10. Configuring ExpressRoute Gateway

Gateway Requirements

- Dedicated GatewaySubnet

- Cannot share subnet

- SKU matters:

  - UltraPerformance / ErGw3AZ → FastPath eligible

11. Provisioning an ExpressRoute Circuit

Provisioning Workflow

- Create circuit in Azure

- Obtain Service Key

- Share key with provider

- Provider completes provisioning

- Circuit becomes Enabled

12. ExpressRoute Peering Configuration

Private Peering Requirements

- Two /30 subnets (primary + secondary)

- VLAN ID

- ASN

- Optional MD5

Microsoft Peering Extras

- Public IP ownership

- Route filters

- Registered prefixes

13. Connecting ExpressRoute to VNets

Connection Flow

- ExpressRoute circuit ↔ Gateway

- Gateway ↔ VNet

- Supports multiple VNets (SKU dependent)

14. ExpressRoute Global Reach

Purpose

- Connect on-premises sites through Microsoft backbone

- Reduces dependency on private WAN

Key Requirement

- ExpressRoute Premium

15. ExpressRoute FastPath

What FastPath Does

- Bypasses VNet gateway

- Traffic flows directly to VM NIC

- Improves:

  - PPS

  - CPS

  - Latency

Requirements

- UltraPerformance or ErGw3AZ gateway

- Enabled via PowerShell

16. Troubleshooting ExpressRoute

Common Checks

- Circuit provisioning state

- Peering status

- BGP routes

- ARP tables

- Gateway health

Tools

- Azure Portal

- PowerShell / CLI

- Azure Network Watcher

- Azure Connectivity Toolkit (AzureCT)

17. ARP Validation (Layer-2 Checks)

ARP helps validate:

- MAC-IP mapping

- L2 connectivity

- Provider misconfigurations

18. Review & Exam Tips

Exam-Focused Takeaways

- ExpressRoute ≠ encrypted by default

- FastPath bypasses gateway

- Premium SKU enables Global Reach

- BFD improves convergence

- Gateway SKU impacts performance